User Guide for NAO Kinect

By: Jill Mercer and Brady Rainey

Group 3

I – Setup:

To run the Kinect Tracking program the computer must have:

- Kinect for Windows SDK installed. (1.8 for Kinect 1.0, 2.0 for Kinect 2.0.)

- A Kinect plugged into the computer.

- On a network with at least one NAO robot.

This guide will help you set up these prerequisites and then get the program up and running.

II – Installing Kinect SDK:

- Navigate to https://developer.microsoft.com/en-us/windows/kinect 	using a web browser.

- Based on your Kinect, pick version 1.8 or version 2.0.

- Follow the installation guide for your version.

III – Connecting Kinect to PC:

- Plug Kinect power into wall.

- Plug Kinect USB into the PC.

- Go to the Start Menu and type in “Device Manager”.

- Look for “Kinect for Windows”.

- Kinect is successfully installed.

- Test by running “Run_KinectViewerDemo” in
the Kinect Tester folder.

IV – Connecting NAO to Your Network:

Wired:

- Remove the plate from the back of NAO’s head.

- Connect an Ethernet cable into the Ethernet port located under this plate.

- Locate your internet router and connect the other end to a free port.

- Power on NAO and it will connect automatically to your network.

V - Running the Program:

- Once setup is complete, launch the NAO program.

- Connect to a NAO by clicking File ➡ Connect and then Select All once connected NAOs are found.

- Select a NAO and then click Run ➡ Kinect Tracking to launch the Kinect Viewer.

- Make sure Kinect has found a skeleton. This is shown by a red stick figure overlay on a person in the Kinect Viewer.

- Once a skeleton is present, the front button on the NAO’s head will begin mirroring. NAO will confirm verbally when mirroring has started.

- The button on the back of the NAO’s head will end mirroring.

VI – Kinect GUI Options:

- Depth Stream

  - Resolution of depth mapping input.

- Mask Players

  - Strips out background.

- Near Mode

  - Changes calculations for closer objects.

- Seated Skeleton

  - Only pulls upper torso of skeletons.

- Texture Stream

  - Resolution of textured input.

- Infrared

  - Show’s input from Kinect’s infrared camera.

- Show Texture

  - Shows color input from Kinect. (Default is the depth mapping.)

- Angle Slider

  - Adjust angle of Kinect. Kinect will angle itself after adjustment.

- Turn Off

  - Turn Off Kinect.